Project 1 Report
Matthew Jackson

1 Spec and Rationale
The goal of this lab is to build a cache coherency system to work with multiple levels of cache, multiple cores, and multiple core complexs. 

The target cache organization I used was provided and not much was deviated against. I did not use a two-phase clocking in this, but I do see how it could be used to sync up timings better. I have 4 cores, 2 core complexes, each core complex is two L1s, and 1 L2. L1 is direct mapped, L2 is Four way set associative. All caches are write-back and Write-allocate. L2 being 4 way is important as it means it does not evict any cache line if one of the L1 request data that has the same tag as the other L1. 
The Data buses are bidirectional, and only one cache can drive the bus at the time. Signals to the arbiter and snooper can happen at any time. The bus for this processor is 32 bits, so a 128bit burst has to be done with blocking. My snooper sits in parallel with the arbiters.

2 Microarchitecture and Cycle Behavior
2.1 Top-Level Block Diagram

2.2 Module Description
Module: Top

Behavior:
Integrates two coherent complexes (CCL0/CCL1), a shared memory arbiter, and an inter-CCL snoop controller.

Consumes CPU0–CPU3 request channels and memory mem_ready/mem_rdata; produces CPU responses and memory bus requests.

Correct means: coherent visibility across CCLs (invalidate/downgrade/writeback paths work), no deadlock, and arbitration serializes shared resources.

Keysignals:   cpu*_req/we/addr/wdata, cpu*_rdata/ready/miss, mem_valid/we/addr, l2_snoop_*, l2_invalidate/l2_downgrade, l2_writeback.

Key state:
Mostly wiring/integration; state is inside instantiated submodules.

Implementation snippet:
// ...existing code...

CCL ccl0 ( /* cpu0/cpu1 + L2 snoop + mem */ );

CCL ccl1 ( /* cpu2/cpu3 + L2 snoop + mem */ );

snoop_controller #(.NUM_CACHES(2), .DATA_WIDTH(32)) l2_snoop ( /* inter-CCL coherence */ );

bus_arbiter #(.NUM_REQUESTORS(2)) mem_arbiter ( /* CCL0/CCL1 -> memory */ );

// ...existing code...

Module: CCL

Behavior:
Implements one core complex: 2×L1 + 1×shared L2 + local L1 snoop + local arbiter.
Consumes two CPU interfaces and external L2 coherence controls; produces CPU responses and memory traffic from its L2.
Correct means: L1-L1 coherence within complex, fair L1 arbitration, and proper forwarding of cross-CCL coherence events to L2/L1.

Key state:
Primarily submodule state; CCL itself is composition/glue.

Implementation snippet:

// ...existing code...

snoop_controller #(.NUM_CACHES(2), .DATA_WIDTH(32)) l1_snoop ( /* L1 coherence */ );

bus_arbiter #(.NUM_REQUESTORS(2)) l1_l2_arbiter ( /* L1a/L1b -> L2 */ );

cachel2_top l2 (

.cpu_req(arb_l2_valid), .cpu_we(arb_l2_we), .cpu_rwitm(arb_l2_rwitm),

.cpu_addr(arb_l2_addr), .cpu_ready(arb_l2_ready),

.l1_has_dirty_data(l1_snoop_dirty), .wait_for_peer_wb(l2_wait_for_peer_wb)

);

// ...existing code...

Module: cachel1_top

Behavior:
Wraps L1 controller + datapath. Handles CPU hits/misses, line fills/evictions, MESI update, snoop invalidate/downgrade.
Consumes CPU request + bus/memory responses + snoop commands; produces CPU response and L1->L2 bus requests.
Correct means: hit detection/address decode correct, refill/write ordering correct, MESI transitions valid.

Key signals: cpu_req/we/addr/wdata,  cpu_req/we/addr/wdata, cpu_rdata/ready/miss, mem_valid/we/rwitm/addr/rdata, mesi_update/mesi_state_next/mesi_addr, snoop_req/invalidate/downgrade.

Key state:
Controller FSM/latched request state in cachel1_controller arrays in cachel1_datapath.

Implementation snippet:

// ...existing code...

cachel1_controller ctrl ( /* FSM + MESI */ );

cachel1_datapath   dp   ( /* arrays + hit/data */ );

assign cpu_miss = miss;

assign cpu_rdata = use_refill_for_cpu ? refill_word : datapath_rdata;

// ...existing code...

Module: cachel1_controller

Behavior:
L1 FSM for compare/upgrade/writeback/allocate and MESI transitions.
Consumes hit/mesi_state, mem_ready, snoop metadata (shared_copy_exists, wait_for_peer_wb), CPU request; drives L1 datapath controls and bus transactions.
Correct means: protocol-safe transitions (I/S/E/M), proper blocking-bus sequencing, and stable request latching.

Key signals: state, word_count, refill_buffer, req_latched, latched_addr, latched_we, mem_valid/we/rwitm/addr, write_enable_word/line, mesi_update.

Key state:

state: current FSM state.

word_count: progress through burst words.

refill_buffer: temporary line buffer during fill.

req_latched, latched_addr, latched_we, latched_shared: frozen request metadata for multi-cycle ops

Implementation snippet:

// ...existing code...

typedef enum logic [2:0] { IDLE, COMPARE_TAG, UPGRADE, WB_ADDR, WB_DATA, ALLOC_ADDR, ALLOC_DATA } state_t;

// ...existing code...

always_ff @(posedge clk or posedge reset) begin

if (reset) begin

state <= IDLE; word_count <= 2'b00; refill_buffer <= 128'b0;

req_latched <= 1'b0; latched_addr <= 32'h0; latched_we <= 1'b0;

end else begin

state <= next_state; word_count <= word_count_next; refill_buffer <= refill_buffer_next;

end

end

// ...existing code...

Module: cachel1_datapath

Behavior: Stores and accesses L1 line/tag/MESI arrays. Computes hit and returns word data; updates arrays on line fill, word write, and MESI events. Consumes controller write enables and addresses; produces hit, rdata, evict_block, mesi_state. Correct means address decode and updates are consistent per index/tag/word. Key signals: tag, index, word_sel, hit, rdata.

Key state:

data_array: cached data words per line.

tag_array: tags per index.

mesi_array: coherence state per index.

Implementation:

// ...existing code...

assign tag   = addr[31:13];

assign index = addr[12:4];

assign hit = (mesi_array[index] != MESI_INVALID) && (tag_array[index] == tag);

if (write_enable_line) begin

data_array[index][0] <= refill_data[31:0];

data_array[index][1] <= refill_data[63:32];

data_array[index][2] <= refill_data[95:64];

data_array[index][3] <= refill_data[127:96];

tag_array[index] <= tag;

end

// ...existing code...

Module: cachel2_top

Behavior: L2 wrapper (controller + set-associative datapath). Handles L1 requests, refill/evict flow, RWITM, and cross-cluster snoop interaction. Consumes arbiter-selected request and memory/snoop inputs; produces L1 response and memory transactions. Correct means correct forwarding path selection and coherence-safe behavior under peer dirty ownership. Key signals: cpu_req, cpu_we, cpu_rwitm, cpu_ready, mem_valid, mem_we, use_refill_data, use_cache_forward, wait_for_peer_wb.

Key state: No significant top-level state beyond submodule-owned state.

Implementation:

// ...existing code...

assign cpu_rdata = use_cache_forward ? datapath_rdata :

use_refill_data   ? (mem_ready ? mem_rdata : forward_rdata) :

datapath_rdata;

cachel2_controller ctrl (/* ... */);

cachel2_datapath dp (/* ... */);

// ...existing code...

Module: cachel2_controller

Behavior: L2 FSM for hit/forward, writeback, allocate/refill, and peer-writeback coordination. Consumes datapath hit/replacement info, snoop/share signals, and memory handshake; produces datapath enables and memory commands. Correct means proper eviction/refill sequencing and no stale-data response during dirty-peer cases. Key signals: state, mem_valid, mem_we, wb_addr, wait_for_peer_wb, latched_addr_out.

Key state:

- state: L2 FSM state.

- word_count: burst progress.

- refill_buffer: temporary fill line.

- latched_addr, latched_we, latched_rwitm, latched_way: captured request/replacement context.

- wait_for_peer_wb: stall until peer dirty writeback completes.

Implementation:

// ...existing code...

typedef enum logic [3:0] {

IDLE, COMPARE_TAG, WB_ADDR, WB_DATA, WB_DONE, WB_DONE2,

ALLOC_ADDR, ALLOC_DATA, FORWARD_DATA, CACHE_FORWARD, RECEIVE_WB

} state_t;

if (latched_rwitm && l1_has_dirty_data) begin

wait_for_peer_wb_next = 1'b1;

end

// ...existing code...

Module: cachel2_datapath

Behavior: 4-way L2 storage datapath with hit detect, replacement metadata, and snoop-read support. Consumes controller writes and snoop ops; produces hit/miss info, selected way data, and snoop visibility outputs. Correct means way/tag/MESI lookups and updates are consistent across sets. Key signals: hit, hit_way, lru_way, snoop_has_line, snoop_data, snoop_mesi_state.

Key state:

- data_array: line data by [way][set][word].

- tag_array: tag by [way][set].

- mesi_array: MESI state by [way][set].

- lru_counter: replacement tracking per set.

Implementation:

// ...existing code...

assign tag   = addr[31:12];

assign index = addr[11:4];

assign snoop_data = data_array[snoop_hit_way][snoop_index][snoop_word_sel];

assign snoop_mesi_state = snoop_has_line ?

mesi_array[snoop_hit_way][snoop_index] : MESI_INVALID;

// ...existing code...

Module: snoop_controller

Behavior: Coherence snoop FSM that gathers peer cache status and issues invalidate/downgrade/writeback controls. Consumes snoop_req, op type, requester id, and peer vectors (cache_hit, cache_dirty, cache_exclusive). Produces coherence commands and response (snoop_ready, shared_copy_exists, snoop_dirty, snoop_dirty_stall). Correct means requester exclusion, stable latched snoop context, and valid state sequencing. Key signals: state, pending_snoop_*, latched_cache_*.

Key state:

- state: snoop FSM state.

- pending_snoop_addr/type/requester_id: active snoop transaction metadata.

- latched_cache_hit/dirty/exclusive: captured peer status for response phase.

Implementation:

// ...existing code...

assign dirty_excluding_requester = active_cache_dirty & ~requester_mask;

assign has_dirty_data = |dirty_excluding_requester;

assign snoop_dirty_stall = (state == SNOOP_CHECK && has_dirty_data) ||

(state == SNOOP_WAIT_DATA);

if (state == SNOOP_CHECK) begin

latched_cache_hit <= cache_hit;

latched_shared_copy_exists <= |cache_hit;

end

// ...existing code...

Module: bus_arbiter

Behavior: Round-robin arbiter for multiple requesters sharing one bus. Consumes vectors of requester valid/control/data and bus_ready; produces one granted bus transaction and per-requester completion/data. Correct means only one active grant, fair grant rotation, and grant hold while transaction is active. Key signals: grant, rr_pointer, transaction_active, granted_id, bus_valid, req_ready.

Key state:

- grant: one-hot current owner.

- rr_pointer: round-robin next-start index.

- req_ready_reg, req_rdata_reg: registered response routing.

Implementation:
// ...existing code...

always_comb begin

bus_valid = 1'b0;

bus_we    = 1'b0;

bus_addr  = 32'b0;

if (|grant) begin

bus_valid = req_valid[granted_id];

bus_we    = req_we[granted_id];

bus_addr  = req_addr[granted_id];

end

end

// ...existing code...

Do you actually want wave forms for 30 tests? Debug verbose is appended in separate .txt, simple test output at end of report.

Text 1: Cold Miss + Allocate
Purpose: Validates that a first read to an uncached line causes a miss, fetches the line from memory, and allocates it into L1 with correct data.
Stimulus: CPU0 issues cpu_read(0x0000_0040) (index 4) using cpu0_req=1, cpu0_we=0; memory model serves a blocking line read over sequential 32-bit words with mem_valid/mem_ready handshake.
Expected result: cpu0_ready eventually asserts, cpu0_miss indicates miss completion, and cpu0_rdata matches initialized memory pattern for address 0x40; L1 index 4 contains the fetched line and valid MESI state.
Checker: Pass if scoreboard does not increment data_mismatches or incorrect_predictions; fail on any mismatch or wrong hit/miss prediction for this transaction.

Text 2: Hit
Purpose: Validates that a second read to the same allocated line is an L1 hit and returns the correct word without miss behavior.
Stimulus: After Test 1 allocation, CPU0 reads 0x0000_0044 (same line, next word).
Expected result: Transaction completes with cpu0_ready=1, hit semantics (cpu0_miss deasserted at completion), and correct cpu0_rdata; line remains resident in L1 index 4.
Checker: Pass if no new scoreboard failures (data_mismatches/incorrect_predictions) are added; otherwise fail.

Text 3: Write Hit (MESI E->M or S->M)
Purpose: Validates write-hit upgrade path and MESI transition to Modified on an already cached line.
Stimulus: CPU0 writes 0xDEADBEEF to 0x0000_0048 (index 4, word2) using cpu0_we=1.
Expected result: Write completes via cpu0_ready; L1 entry at index 4 transitions to MESI 11 (Modified), and written word updates to DEADBEEF.
Checker: Pass if MESI check (DUT.ccl0.l1a.dp.mesi_array[4]==2'b11) and scoreboard checks pass; fail on MESI mismatch or scoreboard error.

Text 4: Miss on Modified Line -> Write-Back -> Allocate
Purpose: Validates dirty eviction from L1 on conflict miss and correct write-back contents into L2 before new allocation.
Stimulus: CPU0 reads 0x0000_2040 (same L1 index 4, different tag) after line 0x40 is Modified.
Expected result: Dirty old line is evicted and written back; new line is allocated; L2 set for 0x40 contains tag/data including updated word DEADBEEF.
Checker: Pass if expected L2 entry is found non-invalid and all four words match {A0000004,B0000004,DEADBEEF,D0000004} and scoreboard has no new failures; otherwise fail.

Text 5: Shared Line (Two Cores Read Same Address)
Purpose: Validates read-sharing rule: two cores reading same line should both hold Shared copies.
Stimulus: CPU0 reads 0x0000_0080, then CPU1 reads 0x0000_0080.
Expected result: Both L1a and L1b at index 8 hold the line in MESI 01 (Shared) with correct data.
Checker: Pass if both MESI states are Shared and no new scoreboard failures; fail otherwise.

Text 6: Invalidation (Core Writes to Shared Line)
Purpose: Validates invalidate-on-write rule for a shared line within same CCL.
Stimulus: Starting from shared 0x80, CPU0 writes 0xCAFEBABE to 0x0000_0080.
Expected result: Writer (CPU0) transitions to Modified, peer (CPU1) is Invalidated, and CPU0 data word updates to CAFEBABE.
Checker: Pass if CPU0 MESI=11, CPU1 MESI=00, written data matches, and scoreboard unchanged; fail on any violation.

Text 7: Modified -> Shared Transition (Intervention)
Purpose: Validates intervention: peer read of a Modified line forces owner downgrade and shares updated data.
Stimulus: CPU1 reads 0x0000_0080 while CPU0 holds it Modified with CAFEBABE.
Expected result: CPU0 and CPU1 both end Shared (01), and both caches contain CAFEBABE for word0.
Checker: Pass if MESI pair is Shared/Shared and both data values match; fail on state or data mismatch.

Text 8: Write-After-Write (Sequential Invalidation)
Purpose: Validates ownership transfer under sequential writes by two cores to same line.
Stimulus: CPU0 and CPU1 first read 0x0000_00C0 (shared), CPU0 writes DEADBEEF, then CPU1 writes FEEDFACE.
Expected result: Final owner is CPU1 Modified (11), CPU0 Invalid (00), final line data reflects second write (FEEDFACE).
Checker: Pass if MESI final states and data match expected and scoreboard unchanged; fail otherwise.

Text 9: Simultaneous Write Arbitration
Purpose: Validates arbiter serialization and coherence correctness when two same-cycle writes target same address.
Stimulus: In one cycle, CPU0 writes AAAAAAAA and CPU1 writes BBBBBBBB to 0x0000_0100, both requests held until each cpu*_ready.
Expected result: One core wins and ends Modified, the other ends Invalid; winner’s line stores its own write data.
Checker: Pass if exactly one MESI is 11, other is 00, and winner data matches; fail on dual-modified/dual-invalid or wrong winner data.

Text 10: Dirty Eviction (Modified Line -> Write-Back -> New Alloc)
Purpose: Validates dirty L1 eviction with sharer history and preservation of modified data in L2.
Stimulus: CPU0 read 0x140, CPU1 read 0x140 (shared), CPU0 write FACADE00, then CPU0 read 0x2140 (same index conflict).
Expected result: CPU0 line evicts; dirty data from 0x140 is present in L2 set 20 with word0 FACADE00; new line allocates at same L1 index.
Checker: Pass if MESI transitions per step are correct and L2 contains expected dirty value for 0x140; fail otherwise.

Text 11: False Sharing (Ping-Pong Write on Same Cache Line)
Purpose: Validates correctness under false sharing: different words in same line written by different cores with invalidation traffic.
Stimulus: CPU0 writes 0x1C0 word0=DEAD0001; CPU1 writes 0x1C4 word1=BEEF0002; both then read back.
Expected result: Final visible line contains both words correctly (word0=DEAD0001, word1=BEEF0002) for both cores after coherence transfers.
Checker: Pass if MESI transitions and all read-back word checks succeed; fail on any word mismatch or invalid state transition.

Text 12: L1 Miss, L2 Hit (Clean)
Purpose: Validates clean victim behavior: line evicted from L1 but retained in inclusive L2, then re-served without memory access.
Stimulus: CPU0 reads A=0x300, then B=0x2300 (same L1 index), then re-reads A; capture mem_transaction_count before/after final read.
Expected result: Re-read A is L1 miss/L2 hit; L1 data restored correctly; no increment in main-memory transaction count on final access.
Checker: Pass if A is found in L2 after eviction, data matches memory pattern, and mc_after==mc_before; fail otherwise.

Text 13: L1 Miss, L2 Hit (Dirty)
Purpose: Validates dirty victim retained in L2 and later served on L1 miss without memory transaction.
Stimulus: CPU0 reads C=0x400, writes 1234ABCD, reads D=0x2400 (evict C from L1 to L2), then re-reads C with mem_transaction_count sampled.
Expected result: L2 stores dirty C with word0 1234ABCD; re-read C returns dirty value from L2; no new memory transaction for final read.
Checker: Pass if dirty C found in L2 with correct data and mc_after==mc_before on step 4; fail on data/state/transaction mismatch.

Text 14: L1 Miss, L2 Miss (L2 Allocation Verified)
Purpose: Validates cold miss allocation correctness in both L1 and L2, including all 4 words and legal MESI post-fill state.
Stimulus: CPU0 reads E=0x500, then inspects L2 set 80 and L1 index 80 arrays.
Expected result: L2 contains tag/data matching memory block at 0x500, MESI is valid (E or S), and L1 also allocated non-invalid with correct tag.
Checker: Pass if any way in L2 set 80 matches tag/data/MESI criteria and L1 allocation checks pass; fail otherwise.

Text 15: L2 Capacity Conflict (4-way Full + Dirty LRU Eviction)
Purpose: Validates 4-way set conflict handling, dirty LRU eviction to memory, and correct transaction accounting.
Stimulus: Access A0/A1/A2/A3/A4 all mapping to L2 set 32; A0 is dirtied first (BB000001) then forced out on A4 access; sample mem_transaction_count and mem_wb_count.
Expected result: On final conflict, exactly one write-back and one fill occur (+1 WB, +2 total mem txns), A0 absent in L2, A4 present, memory word for A0 updated.
Checker: Pass if all set occupancy/tag checks and memory counters/data checks pass; fail on any missing eviction/fill/data condition.

Text 16: Back-to-Back Snoops (Simultaneous CPU0/CPU1 Reads)
Purpose: Validates overlapping same-address read misses are serialized safely and both endpoints converge to Shared with correct data.
Stimulus: fork/join: CPU0 and CPU1 concurrently read 0x7400 in same time window.
Expected result: Both L1 caches at index 320 contain A0000740 and MESI 01 (Shared) after completion.
Checker: Pass if both data checks and both MESI checks succeed, with no new scoreboard failures; fail otherwise.

Text 17: Snoop During L2 Allocation (CPU1 Mid-Fetch)
Purpose: Validates snoop handling while L2 is in active allocate/data phase (request arrives mid-fill).
Stimulus: CPU0 reads 0x7800; CPU1 reads same address after 5 cycles (during ALLOC_DATA window).
Expected result: CPU1 waits until data is available; both caches receive correct data A0000780 and settle in Shared state.
Checker: Pass if both CPUs hold correct word0 and MESI 01/01; fail on stale data, wrong state, or scoreboard error.

Text 18: Modified->Invalid Transition (RWITM Snoop)
Purpose: Validates RWITM semantics: owner with Modified data is invalidated and writer gains exclusive modified ownership.
Stimulus: CPU0 read/write 0x1900 (CAFE0018), then CPU1 writes 0x1900 (FACE0018).
Expected result: CPU0 becomes Invalid, CPU1 becomes Modified with FACE0018 in word0 and preserved non-written word1 from updated line; L2 copy is invalidated after RWITM forward.
Checker: Pass if CPU0 MESI=00, CPU1 MESI=11, CPU1 data words match expected, and no valid L2 copy remains for that tag/set; fail otherwise.

Text 19: Multi-Level Write-Back (L1 -> L2 -> Memory)
Purpose: Validates end-to-end dirty propagation through both cache levels to memory under capacity pressure.
Stimulus: Dirty X=0x0C00 in L1, evict to L2 via Y access, fill set with F1/F2, then F3 forces dirty L2 eviction of X; monitor memory transaction count.
Expected result: X appears dirty in L2 after L1 eviction, later disappears from L2 after L2 eviction, and memory holds DADA0019; final eviction triggers WB+fill behavior.
Checker: Pass if L2 intermediate/final presence checks and memory value check pass, plus expected memory transaction delta; fail otherwise.

Text 20: Cold Write Miss (RWITM from Invalid)
Purpose: Validates write miss from Invalid directly allocates/fills/merges write and lands in Modified without prior read.
Stimulus: CPU0 performs single write 0x0D80 = C01D0020 from cold state.
Expected result: L1 index 216 becomes Modified with word0=C01D0020; untouched words preserve fill data (e.g., word1=B00000D8); inclusive L2 allocation exists.
Checker: Pass if MESI, written word, preserved neighbor word, and L2 presence checks all pass; fail otherwise.

Text 21: L2 Dirty Hit on Write Miss (L2 Serves RWITM)
Purpose: Validates L1 write miss served by dirty line already in L2 (no memory access), with proper merge and Modified result in L1.
Stimulus: Create dirty A=0x0E00 in L2 by writing then evicting from L1 via B=0x2E00; then CPU0 writes A again (2AD00021) while counting memory transactions.
Expected result: Final write is serviced from L2 dirty hit, L1 ends Modified with new word0 and preserved word1; no increase in mem_transaction_count on final step.
Checker: Pass if L2 precondition (dirty A present), L1 post-write data/state, and no-memory-access check all pass; fail otherwise.

Text 22: Cross-CCL Read-Read (Both CCLs Shared)
Purpose: Validates inter-CCL read sharing and downgrade signaling through top-level snoop controller.
Stimulus: CPU0 (CCL0) reads 0x5500, then CPU2 (CCL1) reads same address; allow settle cycles.
Expected result: CCL0 L2 transitions from Exclusive to Shared; CCL1 L2 allocates Shared; both L1s have correct data A0000550.
Checker: Pass if both L2 MESI values are 01 and both L1 data checks pass; fail otherwise.

Text 23: Cross-CCL Exclusive->Shared Downgrade
Purpose: Validates explicit cross-CCL downgrade path when second CCL reads a line held Exclusive by first CCL.
Stimulus: CPU0 reads 0x5880 (CCL0 exclusive), then CPU2 reads same address.
Expected result: CCL0 L2 downgrades E->S, CCL1 L2 allocates S, and both L1 endpoints show correct data A0000588.
Checker: Pass if MESI and data conditions for both CCLs pass; fail on missing line, wrong MESI, or wrong data.

Text 24: Cross-CCL Dirty Intervention (Modified->WB->Shared)
Purpose: Validates dirty-owner intervention across CCL boundary, including writeback to memory and correct data visibility to requester.
Stimulus: CPU0 reads then writes 0x5A00 = CA240024 (Modified), then CPU2 reads same line.
Expected result: CPU2 receives modified value, CPU0 no longer Modified, memory updated with dirty value, and CCL1 L2 contains the line.
Checker: Pass if CPU2 data matches, CPU0 MESI is not 11, memory word equals CA240024, and CCL1 L2 line exists; fail otherwise.

Text 25: Multi-CCL Broadcast Invalidation
Purpose: Validates cross-CCL RWITM invalidates all copies in remote CCL (L2 and both local L1s).
Stimulus: CPU0 and CPU1 share 0x6200; CPU2 writes 0x6200 = BEEF0025.
Expected result: CCL0 L2 copy invalid, CCL0 L1a invalid, CCL0 L1b invalid, CPU2 L1 becomes Modified with written data.
Checker: Pass if all invalidation checks plus CPU2 MESI/data checks pass; fail on any remaining valid stale copy or wrong writer state/data.

Text 26: Cross-CCL Arbiter Race
Purpose: Validates simultaneous cross-CCL write contention is serialized and ends with single owner in Modified.
Stimulus: fork/join simultaneous writes to 0x6400: CPU0=AAAA0026, CPU2=BBBB0026.
Expected result: Exactly one of CPU0/CPU2 ends Modified, loser is not Modified, winner data equals its write payload.
Checker: Pass if XOR-Modified condition is true and winner/loser data-state checks pass; fail otherwise.

Text 27: CCL-Internal then Cross-CCL Downgrade Chain
Purpose: Validates chained coherence transitions: intra-CCL M->S intervention followed by cross-CCL sharing.
Stimulus: CPU0 reads+writes 0x6600=DEAD0027, CPU1 reads same line (intra-CCL share), then CPU2 reads same line (cross-CCL share).
Expected result: CPU1 gets dirty-updated data, both CCL L2s become Shared, and all three L1 copies end Shared with DEAD0027.
Checker: Pass if each step’s MESI/data checks pass (CPU0/CPU1/CPU2 L1 states, CCL0/CCL1 L2 states, propagated data); fail otherwise.

Text 28: Dirty L2 Eviction to Memory – Cross-CCL Read
Purpose: Validates dirty line eviction from CCL0 L2 to memory under set overflow and later retrieval by remote CCL via memory path.
Stimulus: CPU0 dirties A0=0x0C80 (DEAD0028), fills same L2 set with A1..A4 to evict A0, then CPU2 reads A0.
Expected result: A0 removed from CCL0 L2, memory updated with DEAD0028, CPU2 fetches that value and holds line Exclusive.
Checker: Pass if A0-not-found in CCL0 L2, memory value check, CPU2 data check, and CPU2 MESI=10 all pass; fail otherwise.

Text 29: Cross-CCL L2 Capacity Eviction Coherency
Purpose: Same as Test 28 but mirrored on CCL1 as producer and CCL0 as consumer to verify symmetry.
Stimulus: CPU2 dirties A0=0x0C90 (DEAD0029), overfills CCL1 L2 set with A1..A4 causing dirty eviction, then CPU0 reads A0.
Expected result: A0 evicted from CCL1 L2, memory stores DEAD0029, CPU0 fetches updated value and ends Exclusive.
Checker: Pass if CCL1 eviction check, memory value check, CPU0 data check, and CPU0 MESI=10 pass; fail otherwise.

Text 30: Inter-CCL Bus Saturation + MESI Cycling (Ping-Pong)
Purpose: Validates repeated ownership handoff across CCLs under rapid alternating writes to one line, with correct invalidation and data each hop.
Stimulus: CPU0 reads 0x0D00, CPU0 writes AABB0030, CPU2 writes CCDD0030, CPU0 writes EEFF0030, with settle cycles between cross-CCL writes.
Expected result: Ownership cycles CCL0->CCL1->CCL0; previous owner invalidated each time; final owner CPU0 is Modified with EEFF0030.
Checker: Pass if each step’s MESI/data checks pass (CPU0 E then M, CPU2 M while CPU0 I, then CPU0 M while CPU2 I) and scoreboard adds no new failures; fail otherwise.

Non Debugverbose, full verbose in attached .txt file

# === TEST 1: Cold Miss + Allocate ===

# [MEM @115] Transaction START: addr=00000040, block_addr=00000040

# [SCOREBOARD PASS] READ @ 00000040: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000040: Data correct: a0000004

# CPU Read Data = a0000004

# --------------------------------------------------

#  L1 Cache Line @ index 4 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 10

#   Data  : d0000004 c0000004 b0000004 a0000004

# --------------------------------------------------

#

# === TEST 2: Hit ===

# [SCOREBOARD PASS] READ @ 00000044: Hit prediction correct ( HIT)

# [SCOREBOARD PASS] READ @ 00000044: Data correct: b0000004

# --------------------------------------------------

#  L1 Cache Line @ index 4 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 10

#   Data  : d0000004 c0000004 b0000004 a0000004

# --------------------------------------------------

# CPU Read Data = b0000004

#

# === TEST 3: Write Hit (MESI: E->M or S->M) ===

# [SCOREBOARD PASS] WRITE @ 00000048: Hit prediction correct ( HIT)

# --------------------------------------------------

#  L1 Cache Line @ index 4 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d0000004 deadbeef b0000004 a0000004

# --------------------------------------------------

# Memory Block @ address 00000040:

#   [16] = a0000004

#   [17] = b0000004

#   [18] = c0000004

#   [19] = d0000004

# [COHERENCY PASS] TEST3: L1 line is Modified (11)

#

# === TEST 4: Miss on Modified Line -> Write-Back -> Allocate ===

# [MEM @495] Transaction START: addr=00002040, block_addr=00002040

# [SCOREBOARD PASS] READ @ 00002040: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002040: Data correct: a0000204

# CPU Read Data = a0000204

# --------------------------------------------------

#  L1 Cache Line @ index 4 (CPU0)

# --------------------------------------------------

#   Tag       : 00001

#   MESI State: 10

#   Data  : d0000204 c0000204 b0000204 a0000204

# --------------------------------------------------

#

# === Verifying Write-Back to L2 ===

# --------------------------------------------------

#  L2 Cache Set @ index 4

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 11

#     Data      : d0000004 deadbeef b0000004 a0000004

#   Way 1:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d0000204 c0000204 b0000204 a0000204

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

# [COHERENCY PASS] TEST4: Write-back data correct in L2 way 0

# Memory Block @ address 00000040:

#   [16] = a0000004

#   [17] = b0000004

#   [18] = deadbeef

#   [19] = d0000004

#

# === TEST 5: Shared Line (Two Cores Read Same Address) ===

# [MEM @705] Transaction START: addr=00000080, block_addr=00000080

# [SCOREBOARD PASS] READ @ 00000080: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000080: Data correct: a0000008

# [CPU0] Read data = a0000008

# --------------------------------------------------

#  L1 Cache Line @ index 8 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 10

#   Data  : d0000008 c0000008 b0000008 a0000008

# --------------------------------------------------

#

# [CPU1] Reading same address 0x80...

# [CPU1] Read data = a0000008

# --------------------------------------------------

#  L1 Cache Line @ index 8 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d0000008 c0000008 b0000008 a0000008

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 8 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d0000008 c0000008 b0000008 a0000008

# --------------------------------------------------

# [COHERENCY PASS] Both L1s have line in Shared state (01)

#

# === TEST 6: Invalidation (Core Writes to Shared Line) ===

# [CPU0] Writing to shared line at 0x80...

# [SCOREBOARD PASS] WRITE @ 00000080: Hit prediction correct ( HIT)

# --------------------------------------------------

#  L1 Cache Line @ index 8 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d0000008 c0000008 b0000008 cafebabe

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 8 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 00

#   Data  : d0000008 c0000008 b0000008 a0000008

# --------------------------------------------------

# [COHERENCY PASS] CPU0 Modified (11), CPU1 Invalidated (00)

# [COHERENCY PASS] CPU0 has written data: cafebabe

#

# === TEST 7: Modified -> Shared Transition (Intervention) ===

# [CPU1] Reading from CPU0's modified line at 0x80...

# --------------------------------------------------

#  L1 Cache Line @ index 8 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d0000008 c0000008 b0000008 cafebabe

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 8 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d0000008 c0000008 b0000008 cafebabe

# --------------------------------------------------

# [COHERENCY PASS] Both CPUs Shared (01, 01) after intervention

# [COHERENCY PASS] Both CPUs have correct data: cafebabe

#

# === TEST 8: Write-After-Write (Sequential Invalidation) ===

# [CPU0] Reading address 0xC0 to establish shared state...

# [MEM @1365] Transaction START: addr=000000c0, block_addr=000000c0

# [SCOREBOARD PASS] READ @ 000000c0: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 000000c0: Data correct: a000000c

# [CPU1] Reading same address 0xC0...

# --------------------------------------------------

#  L1 Cache Line @ index 12 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d000000c c000000c b000000c a000000c

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 12 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d000000c c000000c b000000c a000000c

# --------------------------------------------------

# [CPU0] Writing to shared line at 0xC0...

# [SCOREBOARD PASS] WRITE @ 000000c0: Hit prediction correct ( HIT)

# --------------------------------------------------

#  L1 Cache Line @ index 12 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d000000c c000000c b000000c deadbeef

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 12 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 00

#   Data  : d000000c c000000c b000000c a000000c

# --------------------------------------------------

# [CPU1] Writing to same line at 0xC0 (should invalidate CPU0)...

# --------------------------------------------------

#  L1 Cache Line @ index 12 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 00

#   Data  : d000000c c000000c b000000c deadbeef

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 12 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d000000c c000000c b000000c feedface

# --------------------------------------------------

# [COHERENCY PASS] CPU0 Invalid (00), CPU1 Modified (11)

# [COHERENCY PASS] CPU1 has final written data: feedface

#

# === TEST 9: Simultaneous Write Arbitration ===

# [TEST9] Both CPUs writing to address 0x100 simultaneously...

# [TEST9 @1965] CPU0 write request: addr=00000100, data=aaaaaaaa

# [TEST9 @1965] CPU1 write request: addr=00000100, data=bbbbbbbb

# [MEM @2035] Transaction START: addr=00000100, block_addr=00000100

# [TEST9 @2165] CPU0 write completed

# [TEST9 @2375] CPU1 write completed

# --------------------------------------------------

#  L1 Cache Line @ index 16 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 00

#   Data  : d0000010 c0000010 b0000010 aaaaaaaa

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 16 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d0000010 c0000010 b0000010 bbbbbbbb

# --------------------------------------------------

# [COHERENCY PASS] CPU1 won arbitration: Modified (11), CPU0: Invalid (00)

# [COHERENCY PASS] CPU1 (winner) has correct data: bbbbbbbb

#

# === TEST 10: Dirty Eviction (Modified Line -> Write-Back -> New Alloc) ===

# [TEST10] CPU0 reading 0x140 (cold miss -> Exclusive)...

# [MEM @2495] Transaction START: addr=00000140, block_addr=00000140

# [SCOREBOARD PASS] READ @ 00000140: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000140: Data correct: a0000014

# [TEST10] CPU0 MESI after read: 10 (expect 10=Exclusive)

# [TEST10] CPU1 reading 0x140 (share -> both Shared)...

# --------------------------------------------------

#  L1 Cache Line @ index 20 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d0000014 c0000014 b0000014 a0000014

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 20 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 01

#   Data  : d0000014 c0000014 b0000014 a0000014

# --------------------------------------------------

# [COHERENCY PASS] TEST10 step2: Both Shared (01)

# [TEST10] CPU0 writing 0x140 with FACADE00 (Shared -> Modified)...

# [SCOREBOARD PASS] WRITE @ 00000140: Hit prediction correct ( HIT)

# --------------------------------------------------

#  L1 Cache Line @ index 20 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d0000014 c0000014 b0000014 facade00

# --------------------------------------------------

# [COHERENCY PASS] TEST10 step3: CPU0 Modified (11), CPU1 Invalid (00)

# [TEST10] CPU0 reading 0x2140 (same index, different tag -> evicts 0x140)...

# [MEM @3005] Transaction START: addr=00002140, block_addr=00002140

# [SCOREBOARD PASS] READ @ 00002140: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002140: Data correct: a0000214

# --------------------------------------------------

#  L1 Cache Line @ index 20 (CPU0)

# --------------------------------------------------

#   Tag       : 00001

#   MESI State: 10

#   Data  : d0000214 c0000214 b0000214 a0000214

# --------------------------------------------------

# [COHERENCY PASS] TEST10 step4: New line allocated at index 20, MESI=10

# [COHERENCY PASS] TEST10 step5: Evicted dirty data in L2 way 0: facade00

#

# === TEST 11: False Sharing (Ping-Pong Write on Same Cache Line) ===

# [TEST11] CPU0 writing word[0] at 0x1C0 = DEAD0001...

# [MEM @3215] Transaction START: addr=000001c0, block_addr=000001c0

# [SCOREBOARD PASS] WRITE @ 000001c0: Hit prediction correct (MISS)

# --------------------------------------------------

#  L1 Cache Line @ index 28 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d000001c c000001c b000001c dead0001

# --------------------------------------------------

# [COHERENCY PASS] TEST11 step1: CPU0 Modified (11) after write

# [TEST11] CPU1 writing word[1] at 0x1C4 = BEEF0002 (should invalidate CPU0)...

# --------------------------------------------------

#  L1 Cache Line @ index 28 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 00

#   Data  : d000001c c000001c b000001c dead0001

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 28 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d000001c c000001c beef0002 dead0001

# --------------------------------------------------

# [COHERENCY PASS] TEST11 step2: CPU0 Invalid (00), CPU1 Modified (11)

# [TEST11] CPU0 reading back 0x1C0 (should see DEAD0001)...

# [SCOREBOARD PASS] READ @ 000001c0: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 000001c0: Data correct: dead0001

# [COHERENCY PASS] TEST11 step3: CPU0 reads correct word[0]: dead0001

# [COHERENCY PASS] TEST11 step4: CPU0 sees CPU1's word[1]: beef0002

# [TEST11] CPU1 reading back 0x1C4 (should see BEEF0002)...

# [COHERENCY PASS] TEST11 step5: CPU1 reads correct word[1]: beef0002

# [COHERENCY PASS] TEST11 step6: CPU1 sees CPU0's word[0]: dead0001

#

# === TEST 12: L1 Miss, L2 Hit (Clean) ===

# [TEST12] Step 1: CPU0 reads A=0x300 (cold miss)...

# [MEM @3945] Transaction START: addr=00000300, block_addr=00000300

# [SCOREBOARD PASS] READ @ 00000300: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000300: Data correct: a0000030

# --------------------------------------------------

#  L1 Cache Line @ index 48 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 10

#   Data  : d0000030 c0000030 b0000030 a0000030

# --------------------------------------------------

# --------------------------------------------------

#  L2 Cache Set @ index 48

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 10

#     Data      : d0000030 c0000030 b0000030 a0000030

#   Way 1:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

# [COHERENCY PASS] TEST12 step1: A is in L2 set 48 after cold miss

# [TEST12] Step 2: CPU0 reads B=0x2300 (same L1 idx, evicts A from L1)...

# [MEM @4155] Transaction START: addr=00002300, block_addr=00002300

# [SCOREBOARD PASS] READ @ 00002300: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002300: Data correct: a0000230

# --------------------------------------------------

#  L1 Cache Line @ index 48 (CPU0)

# --------------------------------------------------

#   Tag       : 00001

#   MESI State: 10

#   Data  : d0000230 c0000230 b0000230 a0000230

# --------------------------------------------------

# --------------------------------------------------

#  L2 Cache Set @ index 48

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 10

#     Data      : d0000030 c0000030 b0000030 a0000030

#   Way 1:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d0000230 c0000230 b0000230 a0000230

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

# [COHERENCY PASS] TEST12 step2: A still in L2 after L1 eviction

# [TEST12] Step 3: CPU0 re-reads A=0x300 (L1 miss, L2 hit expected)...

# [SCOREBOARD PASS] READ @ 00000300: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000300: Data correct: a0000030

# --------------------------------------------------

#  L1 Cache Line @ index 48 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 10

#   Data  : d0000030 c0000030 b0000030 a0000030

# --------------------------------------------------

# [COHERENCY PASS] TEST12 step3: L1[48] data correct after L2 hit: a0000030

# [COHERENCY PASS] TEST12 step3: No memory access (L2 hit confirmed, mc=10)

#

# === TEST 13: L1 Miss, L2 Hit (Dirty) ===

# [TEST13] Step 1: CPU0 reads C=0x400 (cold miss)...

# [MEM @4485] Transaction START: addr=00000400, block_addr=00000400

# [SCOREBOARD PASS] READ @ 00000400: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000400: Data correct: a0000040

# [TEST13] Step 2: CPU0 writes C=0x400 = 1234ABCD (Exclusive->Modified)...

# [SCOREBOARD PASS] WRITE @ 00000400: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST13 step2: L1[64] Modified (11)

# [TEST13] Step 3: CPU0 reads D=0x2400 (evicts dirty C to L2)...

# [MEM @4825] Transaction START: addr=00002400, block_addr=00002400

# [SCOREBOARD PASS] READ @ 00002400: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002400: Data correct: a0000240

# --------------------------------------------------

#  L1 Cache Line @ index 64 (CPU0)

# --------------------------------------------------

#   Tag       : 00001

#   MESI State: 10

#   Data  : d0000240 c0000240 b0000240 a0000240

# --------------------------------------------------

# --------------------------------------------------

#  L2 Cache Set @ index 64

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 11

#     Data      : d0000040 c0000040 b0000040 1234abcd

#   Way 1:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d0000240 c0000240 b0000240 a0000240

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

# [COHERENCY PASS] TEST13 step3: Dirty C in L2 way 0, word[0]=1234abcd

# [TEST13] Step 4: CPU0 re-reads C=0x400 (L1 miss, L2 dirty hit expected)...

# [SCOREBOARD PASS] READ @ 00000400: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000400: Data correct: 1234abcd

# --------------------------------------------------

#  L1 Cache Line @ index 64 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 10

#   Data  : d0000040 c0000040 b0000040 1234abcd

# --------------------------------------------------

# [COHERENCY PASS] TEST13 step4: L1[64] has dirty data from L2: 1234abcd

# [COHERENCY PASS] TEST13 step4: No memory access (L2 dirty hit confirmed)

#

# === TEST 14: L1 Miss, L2 Miss (L2 Allocation Verified) ===

# [TEST14] CPU0 reads E=0x500 (cold miss, L1 idx 80, L2 set 80)...

# [MEM @5155] Transaction START: addr=00000500, block_addr=00000500

# [SCOREBOARD PASS] READ @ 00000500: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000500: Data correct: a0000050

# --------------------------------------------------

#  L1 Cache Line @ index 80 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 10

#   Data  : d0000050 c0000050 b0000050 a0000050

# --------------------------------------------------

# --------------------------------------------------

#  L2 Cache Set @ index 80

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 10

#     Data      : d0000050 c0000050 b0000050 a0000050

#   Way 1:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

# [COHERENCY PASS] TEST14: L2 way 0 allocated E with correct data: a0000050 b0000050 c0000050 d0000050

# [COHERENCY PASS] TEST14: L2 MESI=10 (valid allocation state: E or S)

# [COHERENCY PASS] TEST14: L1[80] allocated correctly, tag=00000 MESI=10

#

# === TEST 15: L2 Capacity Conflict (4-way Full + Dirty LRU Eviction) ===

# [TEST15] Step 1: CPU0 reads A0=0x0200 (cold miss)...

# [MEM @5365] Transaction START: addr=00000200, block_addr=00000200

# [SCOREBOARD PASS] READ @ 00000200: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000200: Data correct: a0000020

# --------------------------------------------------

#  L2 Cache Set @ index 32

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 10

#     Data      : d0000020 c0000020 b0000020 a0000020

#   Way 1:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

# [TEST15] Step 2: CPU0 writes A0=0x0200 word[0] = 0xBB000001...

# [SCOREBOARD PASS] WRITE @ 00000200: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST15 step2: L1[32] Modified (11) after write

# [TEST15] Step 3: CPU0 reads A1=0x1200 (fills L2 way1)...

# [MEM @5615] Transaction START: addr=00001200, block_addr=00001200

# [SCOREBOARD PASS] READ @ 00001200: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00001200: Data correct: a0000120

# [TEST15] Step 4: CPU0 reads A2=0x2200 (dirty WB A0->L2, fills way2)...

# [MEM @5915] Transaction START: addr=00002200, block_addr=00002200

# [SCOREBOARD PASS] READ @ 00002200: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002200: Data correct: a0000220

# [COHERENCY PASS] TEST15 step4: A0 in L2 way 0 as Modified (11) after L1 WB

# [COHERENCY PASS] TEST15 step4: A0 dirty data in L2 way 0 word[0]=bb000001

# [TEST15] Step 5: CPU0 reads A3=0x3200 (fills L2 way3, set now full)...

# [MEM @6125] Transaction START: addr=00003200, block_addr=00003200

# [SCOREBOARD PASS] READ @ 00003200: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00003200: Data correct: a0000320

# --------------------------------------------------

#  L2 Cache Set @ index 32

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 11

#     Data      : d0000020 c0000020 b0000020 bb000001

#   Way 1:

#     Tag       : 00001

#     MESI State: 10

#     Data      : d0000120 c0000120 b0000120 a0000120

#   Way 2:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d0000220 c0000220 b0000220 a0000220

#   Way 3:

#     Tag       : 00003

#     MESI State: 10

#     Data      : d0000320 c0000320 b0000320 a0000320

# --------------------------------------------------

# [COHERENCY PASS] TEST15 step5: All 4 L2 ways valid (set full)

# [COHERENCY PASS] TEST15 step5: Tags 0-3 (A0-A3) all present in L2 set 32

# [TEST15] Step 6: CPU0 reads A4=0x4200 (5th conflict -> dirty LRU evict A0 to memory)...

# [MEM @6335] Transaction START: addr=00000200, block_addr=00000200 (WRITE-BACK)

# [MEM @6435] Transaction START: addr=00004200, block_addr=00004200

# [SCOREBOARD PASS] READ @ 00004200: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00004200: Data correct: a0000420

# --------------------------------------------------

#  L2 Cache Set @ index 32

# --------------------------------------------------

#   Way 0:

#     Tag       : 00004

#     MESI State: 10

#     Data      : d0000420 c0000420 b0000420 a0000420

#   Way 1:

#     Tag       : 00001

#     MESI State: 10

#     Data      : d0000120 c0000120 b0000120 a0000120

#   Way 2:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d0000220 c0000220 b0000220 a0000220

#   Way 3:

#     Tag       : 00003

#     MESI State: 10

#     Data      : d0000320 c0000320 b0000320 a0000320

# --------------------------------------------------

# [COHERENCY PASS] TEST15 step6: Exactly 1 write-back to memory (dirty LRU eviction confirmed)

# [COHERENCY PASS] TEST15 step6: 2 memory transactions (WB + fill), mc=19

# [COHERENCY PASS] TEST15 step6: A0 evicted from L2 set 32 (not present)

# [COHERENCY PASS] TEST15 step6: A4 allocated in L2 set 32 (tag=4 found)

# [COHERENCY PASS] TEST15 step6: Memory has dirty WB data word[0]=bb000001

#

# === TEST 16: Back-to-Back Snoops (Simultaneous CPU0/CPU1 Reads) ===

# [TEST16] CPU0 and CPU1 simultaneously read 0x7400 (cold miss, L1 idx 320)...

# [MEM @6645] Transaction START: addr=00007400, block_addr=00007400

# [SCOREBOARD PASS] READ @ 00007400: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00007400: Data correct: a0000740

# [COHERENCY PASS] TEST16: CPU0 L1[320] word[0]=a0000740 (correct)

# [COHERENCY PASS] TEST16: CPU0 L1[320] MESI=Shared (01)

# [COHERENCY PASS] TEST16: CPU1 L1[320] word[0]=a0000740 (correct)

# [COHERENCY PASS] TEST16: CPU1 L1[320] MESI=Shared (01)

#

# === TEST 17: Snoop During L2 Allocation (CPU1 Requests Mid-Fetch) ===

# [TEST17] CPU0 misses 0x7800; CPU1 requests same line 5 cycles later (mid-fetch)...

# [MEM @6945] Transaction START: addr=00007800, block_addr=00007800

# [SCOREBOARD PASS] READ @ 00007800: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00007800: Data correct: a0000780

# [COHERENCY PASS] TEST17: CPU0 L1[384] word[0]=a0000780 (correct)

# [COHERENCY PASS] TEST17: CPU0 L1[384] MESI=Shared (01)

# [COHERENCY PASS] TEST17: CPU1 L1[384] word[0]=a0000780 (correct)

# [COHERENCY PASS] TEST17: CPU1 L1[384] MESI=Shared (01)

#

# === TEST 18: Modified->Invalid Transition (RWITM Snoop) ===

# [TEST18] Step 1: CPU0 reads 0x1900 (cold miss -> Exclusive)...

# [MEM @7245] Transaction START: addr=00001900, block_addr=00001900

# [SCOREBOARD PASS] READ @ 00001900: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00001900: Data correct: a0000190

# [TEST18] Step 2: CPU0 writes 0x1900 word[0] = 0xCAFE0018...

# [SCOREBOARD PASS] WRITE @ 00001900: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST18 step2: CPU0 L1[400] = Modified (11)

# [TEST18] Step 3: CPU1 writes 0x1900 = 0xFACE0018 (RWITM -> invalidate CPU0)...

# [COHERENCY PASS] TEST18 step3: CPU0 L1[400] = Invalid (00)

# [COHERENCY PASS] TEST18 step3: CPU1 L1[400] = Modified (11)

# [COHERENCY PASS] TEST18 step3: CPU1 word[0]=face0018 (correct)

# [COHERENCY PASS] TEST18 step3: CPU1 word[1]=b0000190 (CPU0 dirty WB propagated)

# [COHERENCY PASS] TEST18: L2 correctly invalidated line after RWITM forward

# --------------------------------------------------

#  L1 Cache Line @ index 400 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 00

#   Data  : d0000190 c0000190 b0000190 cafe0018

# --------------------------------------------------

# --------------------------------------------------

#  L1 Cache Line @ index 400 (CPU1)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d0000190 c0000190 b0000190 face0018

# --------------------------------------------------

# --------------------------------------------------

#  L2 Cache Set @ index 144

# --------------------------------------------------

#   Way 0:

#     Tag       : 00001

#     MESI State: 00

#     Data      : d0000190 c0000190 b0000190 cafe0018

#   Way 1:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

#

# === TEST 19: Multi-Level Write-Back (L1 -> L2 -> Memory) ===

# [TEST19] Step 1: CPU0 reads X=0x0C00 (cold miss)...

# [MEM @7735] Transaction START: addr=00000c00, block_addr=00000c00

# [SCOREBOARD PASS] READ @ 00000c00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000c00: Data correct: a00000c0

# [TEST19] Step 2: CPU0 writes X word[0] = 0xDADA0019...

# [SCOREBOARD PASS] WRITE @ 00000c00: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST19 step2: L1[192] = Modified (11)

# [TEST19] Step 3: CPU0 reads Y=0x2C00 (forces dirty WB of X from L1 to L2)...

# [MEM @8075] Transaction START: addr=00002c00, block_addr=00002c00

# [SCOREBOARD PASS] READ @ 00002c00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002c00: Data correct: a00002c0

# [COHERENCY PASS] TEST19 step3: L2 has X as Modified (11) after L1 WB

# [COHERENCY PASS] TEST19 step3: L2 dirty data word[0]=dada0019 (L1->L2 WB correct)

# [TEST19] Step 4: CPU0 reads F1=0x1C00 (fills L2 set 192 way 2)...

# [MEM @8285] Transaction START: addr=00001c00, block_addr=00001c00

# [SCOREBOARD PASS] READ @ 00001c00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00001c00: Data correct: a00001c0

# [TEST19] Step 5: CPU0 reads F2=0x3C00 (fills L2 set 192 way 3 â set now full)...

# [MEM @8495] Transaction START: addr=00003c00, block_addr=00003c00

# [SCOREBOARD PASS] READ @ 00003c00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00003c00: Data correct: a00003c0

# --------------------------------------------------

#  L2 Cache Set @ index 192

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 11

#     Data      : d00000c0 c00000c0 b00000c0 dada0019

#   Way 1:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d00002c0 c00002c0 b00002c0 a00002c0

#   Way 2:

#     Tag       : 00001

#     MESI State: 10

#     Data      : d00001c0 c00001c0 b00001c0 a00001c0

#   Way 3:

#     Tag       : 00003

#     MESI State: 10

#     Data      : d00003c0 c00003c0 b00003c0 a00003c0

# --------------------------------------------------

# [TEST19] Step 6: CPU0 reads F3=0x4C00 (L2 evicts dirty X to memory)...

# [MEM @8705] Transaction START: addr=00000c00, block_addr=00000c00 (WRITE-BACK)

# [MEM @8805] Transaction START: addr=00004c00, block_addr=00004c00

# [SCOREBOARD PASS] READ @ 00004c00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00004c00: Data correct: a00004c0

# [COHERENCY PASS] TEST19 step6: 2 mem transactions (WB + fill)

# [COHERENCY PASS] TEST19 step6: X evicted from L2 set 192

# [COHERENCY PASS] TEST19 step6: memory[0x0C00]=dada0019 (data reached memory)

# --------------------------------------------------

#  L2 Cache Set @ index 192

# --------------------------------------------------

#   Way 0:

#     Tag       : 00004

#     MESI State: 10

#     Data      : d00004c0 c00004c0 b00004c0 a00004c0

#   Way 1:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d00002c0 c00002c0 b00002c0 a00002c0

#   Way 2:

#     Tag       : 00001

#     MESI State: 10

#     Data      : d00001c0 c00001c0 b00001c0 a00001c0

#   Way 3:

#     Tag       : 00003

#     MESI State: 10

#     Data      : d00003c0 c00003c0 b00003c0 a00003c0

# --------------------------------------------------

#

# === TEST 20: Cold Write Miss (RWITM from Invalid) ===

# [TEST20] Step 1: CPU0 writes 0x0D80 = 0xC01D0020 (cold write miss, RWITM from Invalid)...

# [MEM @9015] Transaction START: addr=00000d80, block_addr=00000d80

# [SCOREBOARD PASS] WRITE @ 00000d80: Hit prediction correct (MISS)

# [COHERENCY PASS] TEST20 step1: L1[216] = Modified (11) after cold write

# [COHERENCY PASS] TEST20 step1: L1[216] word[0] = c01d0020 (write merged)

# [COHERENCY PASS] TEST20 step1: L1[216] word[1] = b00000d8 (memory fill preserved)

# [COHERENCY PASS] TEST20 step1: L2 set216 has line (MESI=11) â inclusive alloc OK

# --------------------------------------------------

#  L1 Cache Line @ index 216 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d00000d8 c00000d8 b00000d8 c01d0020

# --------------------------------------------------

# --------------------------------------------------

#  L2 Cache Set @ index 216

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 11

#     Data      : d00000d8 c00000d8 b00000d8 a00000d8

#   Way 1:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

#

# === TEST 21: L2 Dirty Hit on Write Miss (L2 Serves RWITM) ===

# [TEST21] Step 1: CPU0 reads A=0x0E00 (cold miss -> Exclusive)...

# [MEM @9225] Transaction START: addr=00000e00, block_addr=00000e00

# [SCOREBOARD PASS] READ @ 00000e00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000e00: Data correct: a00000e0

# [TEST21] Step 2: CPU0 writes A=0x0E00 = 0x15100021 (E->M)...

# [SCOREBOARD PASS] WRITE @ 00000e00: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST21 step2: L1[224] = Modified (11)

# [TEST21] Step 3: CPU0 reads B=0x2E00 (evicts dirty A from L1 to L2)...

# [MEM @9565] Transaction START: addr=00002e00, block_addr=00002e00

# [SCOREBOARD PASS] READ @ 00002e00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002e00: Data correct: a00002e0

# [COHERENCY PASS] TEST21 step3: Dirty A in L2 set224, word[0]=15100021

# [TEST21] Step 4: CPU0 writes A=0x0E00 = 0x2AD00021 (L1 miss, L2 dirty hit RWITM)...

# [SCOREBOARD PASS] WRITE @ 00000e00: Hit prediction correct (MISS)

# [COHERENCY PASS] TEST21 step4: L1[224] = Modified (11) after L2 dirty hit RWITM

# [COHERENCY PASS] TEST21 step4: L1[224] word[0] = 2ad00021 (second write merged)

# [COHERENCY PASS] TEST21 step4: L1[224] word[1] = b00000e0 (L2 dirty fill preserved)

# [COHERENCY PASS] TEST21 step4: No memory access (L2 dirty hit confirmed)

# --------------------------------------------------

#  L1 Cache Line @ index 224 (CPU0)

# --------------------------------------------------

#   Tag       : 00000

#   MESI State: 11

#   Data  : d00000e0 c00000e0 b00000e0 2ad00021

# --------------------------------------------------

# --------------------------------------------------

#  L2 Cache Set @ index 224

# --------------------------------------------------

#   Way 0:

#     Tag       : 00000

#     MESI State: 00

#     Data      : d00000e0 c00000e0 b00000e0 15100021

#   Way 1:

#     Tag       : 00002

#     MESI State: 10

#     Data      : d00002e0 c00002e0 b00002e0 a00002e0

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

#

# === TEST 22: Cross-CCL Read-Read (Both CCLs Shared) ===

# [TEST22] Step 1: CPU0 reads 0x5500 (CCL0 cold miss -> Exclusive)...

# [MEM @9895] Transaction START: addr=00005500, block_addr=00005500

# [SCOREBOARD PASS] READ @ 00005500: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00005500: Data correct: a0000550

# [COHERENCY PASS] TEST22 step1: CCL0 L2 set80 Exclusive (10)

# [TEST22] Step 2: CPU2 reads 0x5500 (CCL1 cold miss, cross-CCL snoop -> both Shared)...

# [MEM @10105] Transaction START: addr=00005500, block_addr=00005500

# [COHERENCY PASS] TEST22 step2: CCL0 L2 set80 = Shared (01) after cross-CCL read

# [COHERENCY PASS] TEST22 step2: CCL1 L2 set80 = Shared (01) after cross-CCL read

# [COHERENCY PASS] TEST22: CPU0 L1[336] word[0]=a0000550 (correct)

# [COHERENCY PASS] TEST22: CPU2 L1[336] word[0]=a0000550 (correct)

#

# === TEST 23: Cross-CCL Exclusive->Shared Downgrade ===

# [TEST23] Step 1: CPU0 reads 0x5880 (CCL0 cold miss -> Exclusive)...

# [MEM @10345] Transaction START: addr=00005880, block_addr=00005880

# [SCOREBOARD PASS] READ @ 00005880: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00005880: Data correct: a0000588

# [COHERENCY PASS] TEST23 step1: CCL0 L2 set136 = Exclusive (10)

# [TEST23] Step 2: CPU2 reads 0x5880 (cross-CCL snoop -> CCL0 E->S, CCL1 alloc S)...

# [MEM @10555] Transaction START: addr=00005880, block_addr=00005880

# [COHERENCY PASS] TEST23 step2: CCL0 L2 set136 = Shared (01) â downgraded from E

# [COHERENCY PASS] TEST23 step2: CCL1 L2 set136 = Shared (01) â allocated via cross-CCL read

# [COHERENCY PASS] TEST23: CPU0 L1[392] word[0]=a0000588 (correct)

# [COHERENCY PASS] TEST23: CPU2 L1[392] word[0]=a0000588 (correct)

#

# === TEST 24: Cross-CCL Dirty Intervention (Modified->WB->Shared) ===

# [TEST24] Step 1: CPU0 reads 0x5A00 (CCL0 cold miss -> Exclusive)...

# [MEM @10795] Transaction START: addr=00005a00, block_addr=00005a00

# [SCOREBOARD PASS] READ @ 00005a00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00005a00: Data correct: a00005a0

# [TEST24] Step 2: CPU0 writes 0x5A00 word[0] = 0xCA240024...

# [SCOREBOARD PASS] WRITE @ 00005a00: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST24 step2: CCL0 L1[416] = Modified (11)

# [TEST24] Step 3: CPU2 reads 0x5A00 (cross-CCL dirty intervention)...

# [MEM @11045] Transaction START: addr=00005a00, block_addr=00005a00

# [MEM @11185] Transaction START: addr=00005a00, block_addr=00005a00 (WRITE-BACK)

# [MEM @11275] Transaction START: addr=00005a00, block_addr=00005a00

# [COHERENCY PASS] TEST24 step3: CPU2 L1[416] word[0]=ca240024 (dirty data propagated cross-CCL)

# [COHERENCY PASS] TEST24 step3: CCL0 L1[416] MESI=01 (not Modified â WB completed)

# [COHERENCY PASS] TEST24 step3: memory[0x5A00]=ca240024 (dirty WB reached memory)

# [COHERENCY PASS] TEST24 step3: CCL1 L2 set160 has line (MESI=01)

# --------------------------------------------------

#  L2 Cache Set @ index 160

# --------------------------------------------------

#   Way 0:

#     Tag       : 00005

#     MESI State: 01

#     Data      : d00005a0 c00005a0 b00005a0 ca240024

#   Way 1:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 2:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

#   Way 3:

#     Tag       : xxxxx

#     MESI State: 00

#     Data      : xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx

# --------------------------------------------------

#

# === TEST 25: Multi-CCL Broadcast Invalidation ===

# [TEST25] Step 1: CPU0 reads 0x6200 (CCL0 cold miss -> Exclusive)...

# [MEM @11515] Transaction START: addr=00006200, block_addr=00006200

# [SCOREBOARD PASS] READ @ 00006200: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00006200: Data correct: a0000620

# [TEST25] Step 2: CPU1 reads 0x6200 (intra-CCL snoop -> both Shared)...

# [COHERENCY PASS] TEST25 step2: L1a MESI=01, L1b MESI=01 (both Shared)

# [TEST25] Step 3: CPU2 writes 0x6200=0xBEEF0025 (cross-CCL RWITM)...

# [MEM @11865] Transaction START: addr=00006200, block_addr=00006200

# [COHERENCY PASS] TEST25 step3: CCL0 L2 set32 tag=6 = Invalid

# [COHERENCY PASS] TEST25 step3: CPU0 L1a[32] = Invalid (back-invalidated)

# [COHERENCY PASS] TEST25 step3: CPU1 L1b[32] = Invalid (back-invalidated)

# [COHERENCY PASS] TEST25 step3: CPU2 L1a[32] = Modified (writer)

# [COHERENCY PASS] TEST25 step3: CPU2 data word[0]=beef0025 (correct)

#

# === TEST 26: Cross-CCL Arbiter Race ===

# [TEST26] Issuing simultaneous writes: CPU0=0xAAAA0026, CPU2=0xBBBB0026 to 0x6400...

# [MEM @12105] Transaction START: addr=00006400, block_addr=00006400

# [MEM @12205] Transaction START: addr=00006400, block_addr=00006400

# [MEM @12365] Transaction START: addr=00006400, block_addr=00006400 (WRITE-BACK)

# [MEM @12455] Transaction START: addr=00006400, block_addr=00006400

# [SCOREBOARD PASS] WRITE @ 00006400: Hit prediction correct (MISS)

# [COHERENCY PASS] TEST26: Exactly one Modified â CPU0=11 CPU2=00 (winner: CPU0)

# [COHERENCY PASS] TEST26: Winner CPU0 data=aaaa0026 (correct)

# [COHERENCY PASS] TEST26: Loser CPU2 MESI=00 (not Modified)

#

# === TEST 27: CCL-Internal then Cross-CCL Downgrade Chain ===

# [TEST27] Step 1: CPU0 reads 0x6600 (cold miss -> Exclusive)...

# [MEM @12715] Transaction START: addr=00006600, block_addr=00006600

# [SCOREBOARD PASS] READ @ 00006600: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00006600: Data correct: a0000660

# [TEST27] Step 2: CPU0 writes 0x6600=0xDEAD0027 (E->M silent upgrade)...

# [SCOREBOARD PASS] WRITE @ 00006600: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST27 step2: CPU0 L1a[96] = Modified (11)

# [TEST27] Step 3: CPU1 reads 0x6600 (intra-CCL snoop: CPU0 M->S, CPU1 S)...

# [COHERENCY PASS] TEST27 step3: CPU0 L1a[96] = Shared (01) â downgraded from M

# [COHERENCY PASS] TEST27 step3: CPU1 L1b[96] = Shared (01)

# [COHERENCY PASS] TEST27 step3: CPU1 word[0]=dead0027 (dirty data forwarded)

# [TEST27] Step 4: CPU2 reads 0x6600 (cross-CCL snoop -> all Shared)...

# [MEM @13235] Transaction START: addr=00006600, block_addr=00006600

# [MEM @13285] Transaction START: addr=00006600, block_addr=00006600 (WRITE-BACK)

# [MEM @13375] Transaction START: addr=00006600, block_addr=00006600

# [COHERENCY PASS] TEST27 step4: CCL0 L2 set96 = Shared (01)

# [COHERENCY PASS] TEST27 step4: CCL1 L2 set96 = Shared (01)

# [COHERENCY PASS] TEST27 step4: CPU2 word[0]=dead0027 (dirty data from CCL0)

# [COHERENCY PASS] TEST27 step4: All 3 CPUs Shared â CPU0=01 CPU1=01 CPU2=01

#

# === TEST 28: Dirty L2 Eviction to Memory â Cross-CCL Read ===

# [TEST28] Step 1: CPU0 reads 0x0C80 (cold miss -> Exclusive)...

# [MEM @13615] Transaction START: addr=00000c80, block_addr=00000c80

# [SCOREBOARD PASS] READ @ 00000c80: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000c80: Data correct: a00000c8

# [TEST28] Step 2: CPU0 writes 0x0C80 = 0xDEAD0028 (E->M)...

# [SCOREBOARD PASS] WRITE @ 00000c80: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST28 step2: CPU0 L1a[200] = Modified (11)

# [TEST28] Step 3: CPU0 reads 0x1C80 (fill L2 set200 way1)...

# [MEM @13865] Transaction START: addr=00001c80, block_addr=00001c80

# [SCOREBOARD PASS] READ @ 00001c80: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00001c80: Data correct: a00001c8

# [TEST28] Step 4: CPU0 reads 0x2C80 (L1 evicts dirty A0 to L2, fill L2 way2)...

# [MEM @14165] Transaction START: addr=00002c80, block_addr=00002c80

# [SCOREBOARD PASS] READ @ 00002c80: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00002c80: Data correct: a00002c8

# [TEST28] Step 5: CPU0 reads 0x3C80 (fill L2 set200 way3, set full)...

# [MEM @14375] Transaction START: addr=00003c80, block_addr=00003c80

# [SCOREBOARD PASS] READ @ 00003c80: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00003c80: Data correct: a00003c8

# [TEST28] Step 6: CPU0 reads 0x4C80 (L2 evicts dirty A0 to memory)...

# [MEM @14585] Transaction START: addr=00000c80, block_addr=00000c80 (WRITE-BACK)

# [MEM @14685] Transaction START: addr=00004c80, block_addr=00004c80

# [SCOREBOARD PASS] READ @ 00004c80: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00004c80: Data correct: a00004c8

# [COHERENCY PASS] TEST28 step6: A0 (tag=0) evicted from CCL0 L2 set200

# [COHERENCY PASS] TEST28 step6: memory[0x320] = dead0028 (dirty WB reached memory)

# [TEST28] Step 7: CPU2 reads 0x0C80 (fetch from memory -> must see dirty data)...

# [MEM @14945] Transaction START: addr=00000c80, block_addr=00000c80

# [COHERENCY PASS] TEST28 step7: CPU2 L1a[200] word[0] = dead0028 (dirty data from memory)

# [COHERENCY PASS] TEST28 step7: CPU2 L1a[200] = Exclusive (10)

#

# === TEST 29: Cross-CCL L2 Capacity Eviction Coherency ===

# [TEST29] Step 1: CPU2 reads 0x0C90 (cold miss -> Exclusive)...

# [MEM @15185] Transaction START: addr=00000c90, block_addr=00000c90

# [TEST29] Step 2: CPU2 writes 0x0C90 = 0xDEAD0029 (E->M)...

# [COHERENCY PASS] TEST29 step2: CPU2 L1a[201] = Modified (11)

# [TEST29] Step 3: CPU2 reads 0x1C90 (fill L2 set201 way1)...

# [MEM @15435] Transaction START: addr=00001c90, block_addr=00001c90

# [TEST29] Step 4: CPU2 reads 0x2C90 (L1 evicts dirty A0 to L2, fill L2 way2)...

# [MEM @15735] Transaction START: addr=00002c90, block_addr=00002c90

# [TEST29] Step 5: CPU2 reads 0x3C90 (fill L2 set201 way3, set full)...

# [MEM @15945] Transaction START: addr=00003c90, block_addr=00003c90

# [TEST29] Step 6: CPU2 reads 0x4C90 (L2 evicts dirty A0 to memory)...

# [MEM @16155] Transaction START: addr=00000c90, block_addr=00000c90 (WRITE-BACK)

# [MEM @16255] Transaction START: addr=00004c90, block_addr=00004c90

# [COHERENCY PASS] TEST29 step6: A0 (tag=0) evicted from CCL1 L2 set201

# [COHERENCY PASS] TEST29 step6: memory[0x324] = dead0029 (dirty WB reached memory)

# [TEST29] Step 7: CPU0 reads 0x0C90 (fetch from memory -> must see dirty data)...

# [MEM @16515] Transaction START: addr=00000c90, block_addr=00000c90

# [SCOREBOARD PASS] READ @ 00000c90: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000c90: Data correct: dead0029

# [COHERENCY PASS] TEST29 step7: CPU0 L1a[201] word[0] = dead0029 (dirty data from memory)

# [COHERENCY PASS] TEST29 step7: CPU0 L1a[201] = Exclusive (10)

#

# === TEST 30: Inter-CCL Bus Saturation + MESI Cycling (Ping-Pong) ===

# [TEST30] Step 1: CPU0 reads 0x0D00 (cold miss -> Exclusive)...

# [MEM @16755] Transaction START: addr=00000d00, block_addr=00000d00

# [SCOREBOARD PASS] READ @ 00000d00: Hit prediction correct (MISS)

# [SCOREBOARD PASS] READ @ 00000d00: Data correct: a00000d0

# [COHERENCY PASS] TEST30 step1: CPU0 L1a[208] = Exclusive (10)

# [TEST30] Step 2: CPU0 writes 0x0D00 = 0xAABB0030 (E->M)...

# [SCOREBOARD PASS] WRITE @ 00000d00: Hit prediction correct ( HIT)

# [COHERENCY PASS] TEST30 step2: CPU0 L1a[208] = Modified (11)

# [COHERENCY PASS] TEST30 step2: CPU0 L1a[208] word[0] = aabb0030

# [TEST30] Step 3: CPU2 writes 0x0D00 = 0xCCDD0030 (RWITM, CCL0->CCL1)...

# [MEM @17005] Transaction START: addr=00000d00, block_addr=00000d00

# [MEM @17145] Transaction START: addr=00000d00, block_addr=00000d00 (WRITE-BACK)

# [MEM @17235] Transaction START: addr=00000d00, block_addr=00000d00

# [COHERENCY PASS] TEST30 step3: CPU0 L1a[208] = Invalid (00) after CPU2 write

# [COHERENCY PASS] TEST30 step3: CPU2 L1a[208] = Modified (11)

# [COHERENCY PASS] TEST30 step3: CPU2 L1a[208] word[0] = ccdd0030

# [TEST30] Step 4: CPU0 writes 0x0D00 = 0xEEFF0030 (RWITM, CCL1->CCL0)...

# [MEM @17475] Transaction START: addr=00000d00, block_addr=00000d00

# [MEM @17615] Transaction START: addr=00000d00, block_addr=00000d00 (WRITE-BACK)

# [MEM @17705] Transaction START: addr=00000d00, block_addr=00000d00

# [SCOREBOARD PASS] WRITE @ 00000d00: Hit prediction correct (MISS)

# [COHERENCY PASS] TEST30 step4: CPU2 L1a[208] = Invalid (00) after CPU0 write

# [COHERENCY PASS] TEST30 step4: CPU0 L1a[208] = Modified (11)

# [COHERENCY PASS] TEST30 step4: CPU0 L1a[208] word[0] = eeff0030

#

# === TEST COMPLETE ===

#

# ============================================

#           TEST RESULTS SUMMARY

# ============================================

# Tests Run:               30

# Tests Passed:            30

# Tests Failed:            0

# Status:                  ALL TESTS PASSED

#

# --- Cache Performance ---

# Total Transactions:      60

# Cache Hits:              14

# Cache Misses:            46

# Hit Rate:                23.33%

# Prediction Accuracy:     100.00% (60/60)

#

# --- Data Verification ---

# Read Operations:         42

# Data Correct:            42

# Data Errors:             0

# Data Accuracy:           100.00% (42/42)

#

# --- Coherency Verification ---

# Coherency Checks:        123

# Coherency Passed:        123

# Coherency Failed:        0

# Coherency Accuracy:      100.00% (123/123)

# ============================================

#

# ** Note: $finish    : C:/My_Designs/MLCcoherent/MLCcoherent/Top/src/cache_top_TB.sv(3928)

#    Time: 17875 ps  Iteration: 1  Instance: /cache_top_tb